Quarch Technology Ltd

QuarchPy Python Package

User Guide

Tuesday, 19 February 2019

# Change History

| 1.0 |  | Beta Release |
| --- | --- | --- |
| 1.2 |  | Changed object model, first full release |
| 1.3.1 |  | Added QIS functions |
| 1.3.2 |  | Fixed bug when locally running QIS |
| 1.3.3 |  | Added additional QIS useful functions |
| 1.3.4 |  | Improved QIS launch mechanism Implemented QIS resampling |
| 1.5.5 |  | Added FIO functions and various bug fixes |

Contents

Change History	2

Introduction	4

Modules Supported	5

Requirements	6

Installation	6

Python	6

QuarchPy Package	6

Linux USB Permissions	8

Basic Setup	9

QuarchPy API Listings	11

Class descriptions	11

Helper functions	18

FIO specific functions	20

Customer support from Quarch	21

Access support from the Quarch website	21

Find discussion topics, support information and testing ideas.	21

# Introduction

Quarch modules can be easily controlled using Python through multiple interface options:

USB

Serial

REST

Telnet

QuarchPy is a python package designed to provide an easy to use API which will work seamlessly over any of the communication options.

The package contains all prerequisites needed as a single PyPI project which can be installed from the python online repository or a downloaded local copy.

This makes it easy to install and keep up to date, while also providing you with full access to the source code if you want to make changes or additions.

The package also includes several additional sections

QIS (Quarch Instrument Server)

- This is a headless Java application which handles all the data and multi-thread intensive work needed to stream high-speed data from Quarch Power Modules.

- While it is possible to control power modules with pure Python code, the efficiency is much poorer and streaming speeds will be limited.

- QIS is automatically started when a ‘QIS’ type connection is requested.

QPS (Quarch Power Studio)

- This is a Java based application for control of the Quarch Power Modules.  Also available for download from our site, a version is supplied here as QPS can be automated from Python and controlled via script.

- If you request a QPS connection, the application will be automatically launched, and you can see the live power data while the automatic capture is running.  This provides a great way to get an overview on the results while running a fully automated test.

# Modules Supported

All breaker/hot-swap modules

All Power modules

All Physical layer switch modules

# Requirements

Windows or Linux PC

Python 2.x or Python 3.x

Quarch USB driver (Windows only)

# Installation

## Python

If Python is not installed on your system, go to https://www.python.org/downloads/ and choose the relevant version for your system.

Under Windows it is sensible to ensure the Python installation directory and PythonXX\Scripts are included in the PATH environment variable. See HERE for instructions on how to do this.

## QuarchPy Package

The Quarch Python package can be installed from the Python web repository (assuming you have internet access) or via the download from our site which contained this document.

### Web Install

From the command line:

>pip install quarchpy

If this fails, your path to “pip” may not be set, you can instead run:

>python –m pip install quarchpy

### Local Install

If you want to install from a downloaded .whl file, download the file from https://pypi.org/manage/project/quarchpy/releases/, navigate to the folder containing the quarchpy-x.y.z-py2.py3-none-any.whl file and run:

>pip install quarchpy-x.y.z-py2.py3-none-any.whl

If this fails, your path to ‘pip’ may not be set, you can instead run:

>python –m pip install quarchpy-x.y.z-py2.py3-none-any.whl

Upgrade

If you already have QuarchPy installed, you will get a failure message.  If you want to upgrade to a new version, you need to add the ‘--upgrade’ command:

>pip install --upgrade quarchpy

The --upgrade command can similarly be used in any of the other examples, to load from a local install folder.

## Linux USB Permissions

Linux systems require administrative rights to run python scripts for modules connected via USB. You can do that by running your script as root (sudo command) or changing the default USB permissions. This is done by creating a text file called Quarch-permissions-usb.rules

For ubuntu systems, you need to enter into that file:

SUBSYSTEM == “usb”, ATTRS{idVendor}==”16d0”, MODE=”0666”

SUBSYSTEM == “usb_device”, ATTRS{idVendor}==”16d0”, MODE=”0666”

For Centos systems, you need:

SUBSYSTEM == “usb”, ATTRS{idVendor}==”16d0”, GROUP=”users”, MODE=”0666”

SUBSYSTEM == “usb_device”, ATTRS{idVendor}==”16d0”, GROUP=”users”, MODE=”0666”

This file needs to be placed in /etc/udev/rules.d

Finally, the system either needs to be restarted or run the command:

>sudo udevadm control -reload

Then reconnect the USB device.

# Basic Setup

We strongly recommend you download a relevant application note to demonstrate the use of the Python API, only the basics are shown here.

Below is a simple example to import and use a Quarch module.

# Imports QuarchPy package
from quarchpy import *

# Open a connection to a module on USB
myDevice = quarchDevice(“usb:QTL1999-01-002”)

For USB connections, you specify a full, or partial serial number, as found on the product label.

For LAN connections (ReST and Telnet) you can specify either an IP address, or NetBIOS name (if supported by your network). The default NetBIOS name for most modules is the serial number:

myDevice = quarchDevice (“telnet:192.168.1.55”)

Or

myDevice = quarchDevice (“telnet:QTL1079-03-137”)

For Serial connections, specify the COM port on windows

myDevice = quarchDevice (“SERIAL:COM4”)

Or the linux serial port ID

myDevice = quarchDevice (“SERIAL:ttyS0”) or you may need
myDevice = quarchDevice (“SERIAL:/dev/ttyS0”)

You can now send commands to the module.  The full command list for each module can be found in its Technical Manual, which can be downloaded from www.quarch.com.

To send the simple ‘hello?’ command, use:

myDevice.sendCommand ("hello?")

This function will return the command response, so you can easily see the result with:

print (myDevice.SendCommand("hello?"))

Finally you must close the connection with

myDevice.closeConnection()

# QuarchPy API Listings

## Class descriptions

quarchDevice

Starts a connection and implements basic control over any Quarch device.

quarchDevice (ConString, [ConType = "PY"], [timeout = "5"])

argument string ConString Specifies the device to connect to by COM port, IP address or USB ID

USB:QTL1177 or USB:QTL1177-02 or USB:QTL1177-02-012 Will connect to the first device found matching the string.

SERIAL:COM4 or SERIAL:ttys0 Linux only: ls /dev/serial/ lists serial connections in your system.

TELNET:192.168.1.102 or TELNET:QTL1079-02-004 You can use NetBIOS name if your networks supports it.

REST:192.168.1.102 or REST:QTL1079-02-004 You can use NetBIOS name if your networks supports it.

argument string ConType Allows additional information on connection type for QIS and similar

QIS Local instance of QIS

QIS:192.168.100.102:9722 QIS:ip:port

QPS

Local instance of QPS

PY Python connection

argument string timeout Specifies a timeout when searching for device

return object quarchDevice

sendCommand(command) Sends a command to the device.

argument string command Torridon command to send

return string Answer from module.

openConnection() Opens a connection with device.

closeConnection() Closes the connection with device and any open communication ports.

qisInterface Implements direct communication with QIS.

qisInterface([host=127.0.0.1], [port=9722] )

argument string host Specify the remote host ip.

argument integer port Specify the remote host port.

return object qisInterface

getDeviceList() Returns a list of devices connected to the QIS interface object.

return list Returns a list of devices connected to the QIS interface object.

quarchPPM Extends quarchDevice to provide additional functions for power modules.

quarchPPM(quarchDevice)

argument quarchDevice object quarchDevice Basic quarchDevice object to use as a power module.

startStream([fileName='streamData.txt'], [fileMaxMB=2000], [streamName ='Stream With No Name'], [streamAverage = None], [releaseOnData = False)]

argument string fileName Base file name for the output files.

argument int fileMaxMB Maximun size of a file before creating a new one (default = 2000).

argument string streamName Name for stream file header (default = "Stream With no Name").

argument string streamAverage Sets the averaging for the stream.

argument boolean releaseOnData

If True, only returns from startStream after th first row of data is returned form the module.

streamResampleMode (resampleRate) Sets the resampling mode.

argument resampleRate off, [x]ms or [x]us

stopStream() Stops stream.

quarchQPS Implements the interface with QPS.

quarchQPS (quarchDevice)

argument quarchDevice object quarchDevice quarchDevice Object.

startStream (directory)

argument path directory Declares where the data from the stream will be located.

return object quarchStream

quarchStream Implements an interface with QPS stream.

quarchStream (quarchQPS)

argument quarchQPS object quarchQPS quarchQPS object connected to QPS.

openConnection() Re-open the connection if it has been closed.

closeConnection() Closes the device connection and any open communication ports.

createChannel (channelName, channelGroup, baseUnits, usePrefix) Creates a new channel in QPS.

argument string channelName Name of channel that will be add to QPS.

argument string channelGroup Name of channel group that the channel will be add to.

argument string baseUnits Unit for the new channel.

argument boolean usePrefix Wether the new channel will automatically use metric prefixes to represent numbers.

hideChannel (channelSpecifier) Hides a channel within QPS.

argument string channelSpecifier The channel name the user wants to hide.

hideAllDefaultChannels Hides as default channels. Only user specified channels will be visible.

showChannel (channelSpecifier) Show a specific channel in QPS.

argument string channelSpecifier The channel name the user wants to show.

addAnnotation (annotationString, [annotationTime=0]) Adds an annotation at a specified point in graph. Annotations will be used to create the statistics table.

argument string annotationString Annotation string that will appear in QPS.

"text to show" Will show this whole string on graph. "<<text> text to show</text> <extraText> text to hide</extraText>>" Add the option to hide part of the string. This can be accessed righ clicking in the annotation.

argument string annotationTime Either UNIX time or as elapsed time since the start of the dataset.

1528370532877 Unix time. e12D13:14:15.167 Specifies 12 days, 13 hours, 14 minutes, 15.167 seconds

addComment (commentString, [commentTime=0]) Adds a comment at a specified point in graph. Comments will be hidden from the statistics table.

argument string commentString Comment string that will appear in QPS.

"text to show" Will show this whole string on graph. "<<text> text to show</text> <extraText> text to hide</extraText>>" Add the option to hide part of the string. This can be accessed righ clicking in the comment

argument string commentTime Either UNIX time or as elapsed time since the start of the dataset.

1528370532877 Unix time. e12D13:14:15.167 Specifies 12 days, 13 hours, 14 minutes, 15.167 seconds

addDataPoint (channelName, groupName, dataValue, [dataPointTime]) Adds a new data point to the specified channel for representation on graph.

argument string channelName Name of channel that will be add to QPS.

argument string groupName Name of channel group that the channel will be add to.

argument int driveTemp Value of the data point.

argument string dataPointTime

1528370532877 Unix time. e12D13:14:15.167 Specifies 12 days, 13 hours, 14 minutes, 15.167 seconds

stopStream() Stops the current stream.

myChannels() Returns the active channel.

return list List of channels currently active in QPS.

## Helper functions

QIS

isQisRunning ([host="localhost"], [port"9722"]) Returns wheter QIS is running or not.

argument string host Specify the remote host ip.

argument integer port Specify the remote host port.

return boolean True if QIS is running or False if not.

startLocalQis ([QisPath=/path/inside/QuarchPy]) Start a copy of QIS.

argument path QisPath Path to a custom QIS folder.

closeQis (host, port) Closes QIS.

argument string host Specify the remote host ip.

argument integer port Specify the remote host port.

QPS

isQpsRunning ([host="localhost"], [port"9722"]) Returns wheter QPS is running or not.

argument string host Specify the remote host ip.

argument integer port Specify the remote host port.

return boolean True if QPS is running or False if not.

startLocalQps ([keepQisRunning=False], args=[]) Starts a new instance of QPS.

argument path QisPath Path to a custom QPS folder.

argument boolean keepQisRunning Will keep QIS running after the execution of the script.

closeQps (host, port)

argument string host Specify the remote host ip.

argument integer port Specify the remote host port.

## FIO specific functions

FIO

runFIO (myStream, mode, fioCallbacks, user_data, [arguments], [file_name]) Runs FIO.

argument qpsStream object myStream Your QPS stream where you want to send the data points to.

argument string mode Selects the execution mode.

file QuarchPy with parse the paraemters from the variable "fioFile". arg QuarchPy will parse the parameters from the variable "arguments". cli FIO will run the same way it would in a shell.

argument dictionary fioCallbacks A dictionary containing all the callbacks for FIO.

argument list user_data List of arguments to return from FIO.

argument dictionary arguments List of arguments to send to FIO.

argument string file_name File with FIO configuration.

GetQpsModuleSelection (QpsConnection) Prompts the user to choose a module.

argument qpsStream object QpsConnection Your QPS connection.

# Customer support from Quarch

There are multiple ways to access the support you need. You can contact us directly or access an extensive range of valuable support materials from http://quarch.com/support.

Contact us direct

Get going quickly and easily, with help direct from the engineers:

Call +44 1343 508 140 or email support@quarch.com during UK office hours.

Our international partners are well trained in the use of our products and can deal with many basic technical queries from within your time zone, if you prefer. Check http://quarch.com/resellers for the contact details of your regional supplier.

## Access support from the Quarch website

You can download up-to-date software and drivers, technical manuals, datasheets and more from our website. To help you get started quickly we provide additional documents, such as examples in Perl, Python and C# and Telnet and Serial instructions.

Key places to visit on the Quarch website

Register your Quarch product to confirm your international warranty: http://quarch.com/product-registration

Download a wide range of documentation, free applications and drivers to help you make the best possible use of your Quarch tools: http://quarch.com/content/downloads

Access the Quarch support forum (http://quarch.com/forum):

## Find discussion topics, support information and testing ideas.

Browse existing topics or login to your user account to ask for information and advice.

Sign up for Quarch Technical Updates to get the most out of your Quarch products. Updates are published approximately once a quarter and include news about the latest features, tools, application notes and software updates. See http://quarch.com/content/sign-quarch-technical-updates.